# SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0707

COURSE OUTCOME COVERED

CO4:Design network topologies star, mesh, bus, and hybrid using networking devices, transmission media, and cabling standards

(BL2,BL6)

Assessment Tool Weightage: 30%

Annexure A

## Industry Problem-Solving Task

- A star topology is ideal for a 20-person startup because all devices connect to a central switch, making the network easy to manage and troubleshoot. At the core sits a Layer 2/Layer 3 managed switch (such as a Cisco Catalyst or Netgear ProSafe) with at least 24 ports to accommodate all workstations, printers, and VoIP phones with room to grow. Each device connects to the switch using Cat6 UTP cables, which support Gigabit speeds up to 100 meters — more than sufficient for any typical office floor plan. The switch uplinks to a router/firewall (such as a Cisco RV series or pfSense appliance) that handles internet access, DHCP, and network address translation. A Wi-Fi access point connected to the switch extends wireless coverage for laptops and mobile devices. The key advantage of this design is fault isolation: if one workstation's cable fails, only that single node goes offline, leaving all other 19 employees unaffected. Centralised cable management also simplifies future moves, adds, and changes.

- A bank demands near-zero downtime, making a full or partial mesh topology the right choice. In a full mesh, every core network device — routers, core switches, and servers — connects directly to every other device, so no single link or node failure can bring down the network. For a bank, a partial mesh is typically deployed at the core layer (connecting all core switches and firewalls to each other) while the distribution and access layers use redundant uplinks to at least two core switches. Dual enterprise-grade routers (such as Cisco ASR or Juniper MX series) provide BGP-based redundant internet connectivity. All links between core devices use fiber optic cables — either single-mode for long distances between buildings or multi-mode OM4 within the data center — because fiber is immune to electromagnetic interference, supports very high bandwidth, and is essential for maintaining low-latency financial transactions. Redundant power supplies, UPS systems, and RAID storage arrays further complement the mesh design. The justification is straightforward: when a link fails in a mesh, traffic automatically reroutes through alternative paths in milliseconds using protocols like OSPF or BGP, satisfying regulatory requirements for uptime and data integrity.

- For a small classroom lab with a tight budget, bus topology offers the lowest hardware cost because all computers share a single common communication line — the bus — eliminating the need for a central switch or hub. A single coaxial cable (RG-58, 50-ohm) or, in a modern variation, a single Cat5e/Cat6 cable daisy-chained through an inexpensive unmanaged hub runs along the length of the room. Each computer connects to the backbone using a T-connector and a BNC connector in a classic coaxial setup, with terminators (50-ohm) placed at both ends of the cable to absorb signals and prevent reflection. For a more contemporary low-cost implementation, a single inexpensive 8- or 16-port unmanaged switch replaces the terminators and uses Cat5e UTP patch cables to each workstation — achieving bus-like simplicity at minimal cost. The primary limitation is that the network is susceptible to collisions and a single cable break disrupts all connected machines, but for a supervised classroom environment with light usage (internet browsing, file sharing), this trade-off is entirely acceptable. No dedicated server is required; a teacher's PC can share resources using built-in Windows/Linux file sharing.

- A large university campus serves thousands of users across multiple buildings — lecture halls, labs, libraries, dormitories, and administration — making a single topology inadequate. A hybrid topology combines the best elements of multiple designs: a fiber-based ring or mesh at the campus backbone level for redundancy, a star topology within each building for manageability, and bus or tree structures within individual labs for cost-efficiency. At the campus core, high-speed fiber optic cables (single-mode, 10 Gbps or 40 Gbps) interconnect core routers and multilayer switches using a ring or partial mesh arrangement, ensuring that no single link failure isolates an entire building. Each building has its own distribution switch connected to the campus core via dual fiber uplinks. Within each building, a traditional star topology branches from the distribution switch to floor-level access switches, which then connect individual devices via Cat6 cables. This hierarchical three-layer model — core, distribution, and access — is the industry standard for enterprise and campus networks because adding a new building simply means installing a new distribution switch and running fiber to the core, with no disruption to existing infrastructure. Wireless access points throughout the campus connect via PoE (Power over Ethernet) to access switches, eliminating the need for separate power cabling. The hybrid design thus delivers redundancy at the backbone, simplicity at the building level, and scalability campus-wide.

- Designing a network for a company with multiple departments involves creating a structured and scalable architecture using switches and routers to ensure efficient communication and security. Each department (such as HR, Finance, Sales, and IT) is connected through dedicated Layer 2 switches that form the access layer, allowing devices like computers and printers to communicate within the same department. These switches are then connected to a central Layer 3 switch or router, which acts as the core layer and enables communication between different departments through inter-VLAN routing. Virtual LANs (VLANs) are configured to logically separate each department’s network, improving security and reducing broadcast traffic. A router is also used to connect the internal network to external networks such as the internet, often with firewall functionality for security. Redundant links and backup paths can be included to ensure reliability, while proper IP addressing and subnetting are applied for efficient network management. This hierarchical design improves performance, enhances security, and allows easy expansion as the company grows.

Question 1 – Star Topology for a 20-Person Startup

1. Industry Problem

A 20-person startup needs a network that is simple to manage, easy to troubleshoot, and capable of future growth. A star topology is suitable because every workstation, printer, and VoIP phone connects to a central managed switch.

2. Proposed Network Design

- Use a 24-port Layer 2/Layer 3 managed switch.

- Connect all devices to the central switch.

- Use Cat6 UTP cables for device connections.

- Keep cable length within 100 m.

- Connect the switch to a router/firewall.

- Use the router for Internet access, DHCP and NAT.

- Add a Wi-Fi access point for wireless devices.

3. Network Structure

Internet

|

Router / Firewall

|

Central Switch

_____________|_____________

|        |       |          |

PC 1      PC 2   Printer    VoIP Phone

|

Cat6 UTP

+ Wi-Fi Access Point

|

Laptops / Mobiles

4. Why Star Topology?

- Centralized network management.

- Easy fault identification.

- Easy to add devices.

- Dedicated connection for each device.

- Better network organization.

- One cable failure affects only one device.

If one workstation cable fails, only that workstation is disconnected, while the remaining 19 employees can continue using the network.

5. Result

The proposed star topology provides a manageable, reliable, and scalable network for the startup. The central managed switch simplifies administration, while Cat6 UTP provides suitable high-speed connections within the office.

Question 2 – Mesh Topology for a Banking Network

- Industry Problem

A bank requires near-zero network downtime because continuous connectivity is important for financial transactions. A full or partial mesh topology is suitable because it provides multiple communication paths and avoids dependence on a single link.

- Proposed Network Design

- Use a partial mesh at the core layer.

- Connect core switches and firewalls redundantly.

- Use two enterprise-grade routers.

- Provide dual uplinks from distribution switches.

- Use fiber optic cables between core devices.

- Use single-mode fiber for long distances.

- Use OM4 multi-mode fiber inside data centers.

- Use OSPF or BGP for routing and rerouting.

3. Network Structure

Internet

/      \

Router 1    Router 2

| \      / |

|  \    /  |

Core Switches

/    |     \

Distribution Switches

|    |    |

Bank Departments

4. Why Mesh Topology?

- Provides multiple network paths.

- Reduces single-point failures.

- Supports automatic traffic rerouting.

- Provides high network availability.

- Fiber provides high bandwidth.

- Fiber is resistant to electromagnetic interference.

- Supports low-latency financial communication.

When one link fails, traffic can automatically use an alternative path through routing protocols such as OSPF or BGP.

5. Result

The mesh-based design provides the bank with high availability, redundancy, and reliable communication. Alternative paths allow network traffic to continue even when a link fails, supporting the bank's requirement for near-zero downtime.

Question 3 – Bus Topology for a Small Classroom Lab

1. Industry Problem

A small classroom lab needs a low-cost network for basic activities such as internet browsing and file sharing. Since the number of computers and network requirements are limited, a bus topology is suitable because multiple computers can share a common communication line with low hardware cost.

2. Proposed Network Design

- Use a common communication backbone.

- Use RG-58 coaxial cable in a classic bus setup.

- Use T-connectors and BNC connectors for computers.

- Place 50-ohm terminators at both cable ends.

- A modern low-cost option can use Cat5e/Cat6 with an inexpensive unmanaged switch.

3. Network Structure

50Ω Terminator

|

|==============================|

|        Common Bus            |

PC1      PC2      PC3      PC4

|        |        |        |

T-       T-       T-       T-

|        |        |        |

|==============================|

|

50Ω Terminator

4. Why Bus Topology?

- Very low hardware cost.

- No central switch required.

- Simple for small labs.

- Suitable for light network usage.

- Supports basic file sharing.

- Suitable for internet browsing.

The main limitation is that a single cable failure can affect all connected computers, and the shared medium is susceptible to collisions.

5. Result

Bus topology is suitable for a small, budget-limited classroom lab with light network usage. It reduces hardware requirements while providing basic connectivity for browsing and file sharing.

Question 4 – Hybrid Topology for a University Campus

1. Industry Problem

A large university campus has thousands of users across lecture halls, laboratories, libraries, dormitories, and administration buildings. A single topology cannot efficiently meet all these requirements. Therefore, a hybrid topology is used to provide redundancy, manageability, and scalability.

2. Proposed Network Design

- Use fiber ring or partial mesh at the campus backbone.

- Use star topology inside each building.

- Use bus or tree structures in laboratories.

- Use 10 Gbps or 40 Gbps fiber at the campus core.

- Connect each building using dual fiber uplinks.

- Use Cat6 cables for floor-level devices.

- Use PoE access points for campus Wi-Fi.

3. Network Structure

Campus Core

Ring / Partial Mesh

/        |        \

Building 1  Building 2  Building 3

|          |           |

Distribution Distribution Distribution

Switch       Switch       Switch

|          |           |

Access       Access       Access

Switches     Switches     Switches

/ | \        / | \        / | \

PCs APs Lab   PCs APs Lab  PCs APs Lab

4. Why Hybrid Topology?

- Provides backbone redundancy.

- Simplifies building-level management.

- Supports large campus expansion.

- Prevents one link failure.

- Provides high-speed backbone connectivity.

- Supports campus-wide wireless access.

The core, distribution, and access structure is suitable for campus networks because a new building can be added by installing a distribution switch and connecting it to the core without disturbing existing buildings.

5. Result

The hybrid topology provides redundancy, manageability, and scalability for a large university campus. Combining ring/mesh, star, and tree/bus structures allows each part of the campus to use a topology suitable for its requirements.

Question 5 – Department-wise Network Design

1. Industry Problem

A company with HR, Finance, Sales, and IT departments needs a structured network for secure and efficient communication. Each department requires separate network access while still communicating with other departments when necessary.

2. Proposed Network Design

- Use a dedicated Layer 2 switch for each department.

- Connect departmental switches to a central Layer 3 switch/router.

- Create separate VLANs for each department.

- Use inter-VLAN routing for department communication.

- Connect the internal network to the Internet through a router.

- Use firewall functionality for network security.

- Apply proper IP addressing and subnetting.

3. Network Structure

Internet

|

Router / Firewall

|

Layer 3 Switch

_________|_________

|         |         |

VLAN-HR   VLAN-Fin   VLAN-Sales

|         |         |

L2 Switch  L2 Switch  L2 Switch

|         |         |

PCs       PCs       PCs

VLAN-IT

|

L2 Switch

|

PCs

4. Why This Design?

- Separates departmental networks.

- Reduces broadcast traffic.

- Improves network security.

- Supports inter-department communication.

- Simplifies network administration.

- Provides better network performance.

- Supports future company expansion.

5. Result

The proposed hierarchical network provides secure, organized, and scalable communication between company departments. VLANs provide logical separation, while the Layer 3 switch or router enables controlled communication between departments.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Problem Analysis | 15% | Clearly identifies and deeply analyzes the industry problem with all factors | Good understanding with minor gaps | Basic understanding; some key aspects missing | Poor or incorrect understanding of the problem |
| Solution Design | 20% | Innovative, practical, and industry-relevant solution | Effective solution with minor limitations | Partially suitable solution | Inappropriate or unclear solution |
| Technical Implementation | 20% | Fully functional, accurate, and well-executed implementation | Mostly functional with minor errors | Partially implemented solution | Incomplete or non-functional implementation |
| Use of Tools & Technologies | 10% | Appropriate and advanced use of relevant tools and technologies | Correct use of tools | Limited or basic use of tools | Incorrect or no use of tools |
| Problem-Solving Approach | 10% | Logical, systematic, and well-structured approach | Mostly logical approach | Somewhat disorganized approach | No clear approach |
| Innovation & Creativity | 10% | Highly creative and original solution | Some creativity | Limited originality | No creativity |
| Testing & Validation | 5% | Thorough testing with real-world scenarios | Adequate testing | Minimal testing | No testing |
| Documentation & Presentation | 5% | Clear, professional, and well-documented | Good documentation | Basic documentation | Poor or no documentation |
| Teamwork / Collaboration | 5% | Excellent coordination and contribution (if team-based) | Good collaboration | Limited participation | No collaboration or contribution |