# SIMATS ENGINEERING

## ASSESSMENT TOOL 3

### COURSE NAME: Computer Networks	COURSE CODE:	CSA0707

COURSE OUTCOME COVERED

CO5: Measure network performance using key metrics with simulation tools

Assessment Tool Weightage: 20%

### QUESTIONS: 40	Total Marks:40

Annexure A Mock Interview

### QUESTIONS:40	Total Marks:40

- Analyze how network congestion affects packet delay and packet loss. (BL4)(1)

- Compare the impact of star and mesh topologies on network reliability and fault tolerance. (BL4)(1)

- Why does increasing the number of routers between source and destination increase end-to-end delay? Explain logically. (BL4)(1)

- Analyze how Quality of Service (QoS) improves the performance of voice and video traffic. (BL4)(1)

- Evaluate the impact of using fiber optic cables instead of copper cables for high-speed communication. (BL5)(1)

- Analyze how network segmentation using VLANs improves performance and security. (BL4)(1)

- Compare the effects of TCP and UDP on network performance for different applications. (BL4)(1)

- Why is load balancing important in large enterprise networks? Analyze its impact on performance and availability. (BL5)(1)

- Analyze how frequent routing updates influence network efficiency and bandwidth utilization. (BL4)(1)

- Evaluate the trade-off between network security measures (such as firewalls and encryption) and overall network performance. (BL5)(1)

- A network has high delay—how will you identify and fix the issue? (BL4)(1)

- If throughput is low despite high bandwidth, what could be the reasons? (BL4)(1)

- How would you reduce packet loss in a congested network? (BL4)(1)

- A VoIP call is breaking frequently—how would you troubleshoot? (BL4)(1)

- Given a network with high jitter, suggest improvements (BL4)(1)

- How will you simulate a congested network in Packet Tracer? (BL5)(1)

- If packets are delayed in a router, what steps will you take? (BL4)(1)

- How would you optimize a network for online gaming? (BL4)(1)

- Design a small network and explain how you will measure its performance (BL5)(1)

- If RTT is very high, how will you diagnose the issue? (BL4)(1)

- Explain network performance to a non-technical person (BL5)(1)

- Describe throughput using a real-life example (BL5)(1)

- Present how packet loss affects video streaming (BL5)(1)

- Explain delay types clearly in under 1 minute (BL4)(1)

- Describe Cisco Packet Tracer in a professional way (BL5)(1)

- Explain QoS in simple and technical terms (BL5)(1)

- How would you present your simulation results to a client? (BL4)(1)

- Explain jitter using a real-world analogy (BL5)(1)

- Describe how you tested network performance in a lab (BL4)(1)

- Explain the importance of performance metrics in one structured answer (BL5)(1)

- Introduce yourself and explain your knowledge of network performance (BL5)(1)

- Why are you interested in networking and simulation tools? (BL5)(1)

- What makes you confident in analyzing network performance? (BL5)(1)

- Explain a concept you are very strong in (related to networking) (BL5)(1)

- How would you handle a question you don’t know? (BL5)(1)

- Describe a situation where you solved a technical problem (BL4)(1)

- Why should we select you for a networking role? (BL5)(1)

- What challenges did you face while learning Packet Tracer? (BL5)(1)

- How do you improve your technical skills regularly? (BL5)(1)

- Demonstrate confidence while explaining a complex concept simply (BL5)(1)

1. How does network congestion affect packet delay and packet loss?

Network congestion occurs when traffic exceeds available network capacity. Packets wait in queues, increasing delay. When buffers become full, packets are dropped, causing packet loss.

2. Compare star and mesh topologies on reliability and fault tolerance.

Star topology is reliable for individual link failures, but central-switch failure affects the network. Mesh provides multiple paths, so communication can continue even when some links fail. Therefore, mesh has higher fault tolerance.

3. Why do more routers increase end-to-end delay?

Every router adds processing, queuing, and transmission time. Therefore, as the number of routers increases, the total end-to-end delay generally increases.

4. How does QoS improve voice and video traffic?

QoS prioritizes delay-sensitive voice and video packets over less-sensitive traffic. It reduces delay, jitter, and packet loss, providing smoother calls and video streaming.

5. Evaluate fiber optic cables instead of copper for high-speed communication.

Fiber provides higher bandwidth, longer transmission distance, lower signal loss, and immunity to electromagnetic interference. Its main disadvantage is higher installation cost compared with copper.

6. How does VLAN segmentation improve performance and security?

VLANs divide a network into logical segments. This reduces unnecessary broadcast traffic and limits communication between groups, improving both network performance and security.

7. Compare TCP and UDP for different applications.

TCP provides reliable, ordered delivery and is suitable for web browsing and file transfer. UDP has lower overhead and faster transmission, making it suitable for streaming, VoIP, and online gaming.

8. Why is load balancing important in large enterprise networks?

Load balancing distributes traffic across multiple servers or links. It prevents overload, improves response time, increases resource utilization, and provides better availability.

9. How do frequent routing updates affect network efficiency?

Frequent routing updates consume bandwidth and processing resources. Excessive updates can reduce efficiency, while timely updates help routers maintain accurate and efficient paths.

10. What is the trade-off between security and network performance?

Firewalls and encryption improve security but require additional processing. This can increase latency and reduce performance slightly. Proper configuration provides security without unnecessary performance loss.

11. A network has high delay. How will you identify and fix it?

I would measure latency using ping and traceroute, identify the problematic link or router, check congestion and device utilization, and then optimize routing, bandwidth, or hardware accordingly.

12. If throughput is low despite high bandwidth, what could be the reasons?

Possible reasons include congestion, packet loss, high latency, protocol overhead, hardware limitations, interference, or poor network configuration.

13. How would you reduce packet loss in a congested network?

I would identify the congested link, reduce unnecessary traffic, increase bandwidth, apply QoS, optimize routing, and use load balancing where required.

14. A VoIP call is breaking frequently. How would you troubleshoot?

I would check packet loss, delay, jitter, bandwidth, and network congestion. Then I would verify QoS settings, connectivity, and VoIP device configuration.

15. Given a network with high jitter, suggest improvements.

I would identify congestion, apply QoS, prioritize real-time traffic, optimize routing, reduce unnecessary traffic, and ensure sufficient bandwidth.

16. How will you simulate a congested network in Packet Tracer?

I would create multiple communicating devices, generate high traffic, limit link bandwidth, and observe delay, packet loss, and throughput using Packet Tracer simulation tools.

17. If packets are delayed in a router, what steps will you take?

I would check router CPU and memory usage, interface utilization, queue length, routing configuration, and packet statistics. Then I would identify and remove the bottleneck.

18. How would you optimize a network for online gaming?

I would reduce latency and jitter, minimize packet loss, prioritize gaming traffic using QoS, select efficient routes, and provide sufficient bandwidth.

19. Design a small network and explain how you will measure its performance.

I would connect PCs through a switch and router. I would measure throughput, delay, packet loss, jitter, and bandwidth utilization using tools such as Packet Tracer.

20. If RTT is very high, how will you diagnose the issue?

I would use ping and traceroute to locate the delay. Then I would check congestion, routing paths, link quality, and router performance.

21. Explain network performance to a non-technical person.

Network performance describes how well a network works. It is mainly measured by how fast data travels, how much data is delivered, and how often data is delayed or lost.

22. Describe throughput using a real-life example.

Throughput is the actual amount of data successfully transferred per second. For example, if a network successfully transfers 50 MB in one second, its throughput is 50 MB/s.

23. How does packet loss affect video streaming?

Packet loss causes missing video data. This can produce buffering, freezing, reduced quality, or interruptions during video streaming.

24. Explain delay types in under one minute.

The main delays are processing, queuing, transmission, and propagation delay. Processing occurs inside devices, queuing occurs while waiting for transmission, transmission depends on packet size and bandwidth, and propagation depends on the physical distance and medium.

25. Describe Cisco Packet Tracer professionally.

Cisco Packet Tracer is a network simulation tool used to design, configure, test, and troubleshoot network topologies without requiring physical networking equipment.

26. Explain QoS in simple and technical terms.

Simply, QoS gives important traffic higher priority. Technically, QoS classifies and prioritizes traffic to control delay, jitter, packet loss, and bandwidth usage.

27. How would you present simulation results to a client?

I would present the network topology, testing method, key performance metrics, results, identified problems, and recommended improvements using clear graphs or summarized data.

28. Explain jitter using a real-world analogy.

Jitter is variation in packet arrival time. For example, if buses are expected every 10 minutes but arrive at irregular intervals, that variation is similar to network jitter.

29. Describe how you tested network performance in a lab.

I created the network topology, configured the devices, generated traffic, and measured throughput, delay, packet loss, jitter, and bandwidth utilization.

30. Explain the importance of performance metrics.

Performance metrics help identify network problems and evaluate network quality. Throughput, delay, packet loss, jitter, and bandwidth utilization help determine whether a network is efficient and reliable.

31. Introduce yourself and explain your knowledge of network performance.

I am a Computer Science student with an interest in computer networking. I understand basic network performance metrics such as throughput, delay, packet loss, jitter, and bandwidth utilization, and I have studied network simulation using Packet Tracer.

32. Why are you interested in networking and simulation tools?

I am interested in networking because it explains how systems communicate. Simulation tools allow me to design and test networks safely before implementing them in real environments.

33. What makes you confident in analyzing network performance?

My understanding of performance metrics and systematic troubleshooting gives me confidence. I analyze the problem, identify the bottleneck, measure the relevant metrics, and then apply a suitable solution.

34. Explain a concept you are very strong in.

I am strong in network performance analysis. I can understand throughput, delay, packet loss, and jitter and use these metrics to identify network performance problems.

35. How would you handle a question you don't know?

I would remain calm and honestly state that I do not know the exact answer. I would explain what I understand about the topic and, if possible, use logical reasoning to approach the problem.

36. Describe a situation where you solved a technical problem.

During a networking lab, if communication failed, I would systematically check the cable connection, IP configuration, subnet, gateway, and routing. By checking each layer, I can identify and correct the problem.

37. Why should we select you for a networking role?

I have a strong interest in networking, a willingness to learn, and a systematic approach to troubleshooting. I can understand technical problems and work toward practical and reliable solutions.

38. What challenges did you face while learning Packet Tracer?

Initially, configuring IP addresses, routing, and understanding device connections were challenging. By practicing different topologies and troubleshooting configurations, I improved my understanding.

39. How do you improve your technical skills regularly?

I improve my skills through regular practice, laboratory exercises, simulation tools, technical documentation, and solving networking problems.

40. Demonstrate confidence while explaining a complex concept simply.

I first identify the main idea and explain it using simple terms and a real-life example. Then I gradually introduce the technical details. This makes the concept easier to understand without losing accuracy.

### RUBRICS FOR CALCULATION

| Criteria | Weightag e (%) | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Conceptual Understandin g | 30% | Demonstrates thorough understandin g; answers all questions accurately | Shows good understandin g with few errors | Partial understandin g; several incorrect responses | Lacks understandin g; majority of answers incorrect |
| Accuracy of Answers | 25% | All answers are correct | Most answers are correct with minor mistakes | Some correct answers; frequent errors | Mostly incorrect answers |
| Application of Knowledge | 20% | Correctly applies concepts to problem-based questions | Applies concepts with minor errors | Limited ability to apply concepts | Unable to apply concepts |
| Time Management | 15% | Completes quiz well within time efficiently | Completes on time with minor delays | Requires extra time or rushes through questions | Unable to complete within given time |
| Question Interpretation | 10% | Interprets all questions correctly and responds appropriately | Misinterprets a few questions | Often misinterprets questions | Unable to understand questions |