# SIMATS ENGINEERING

### ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS COURSE CODE: CSA0707

COURSE OUTCOME COVERED

CO5:

Measure network performance using key metrics with simulation tools

(BL4,BL5)

Assessment Tool Weightage: 40%

QUESTIONS: 3	Total Marks:30

Annexure A

## Constraint-Based Problem Solving

- A multinational company is opening a new branch office in a remote rural location where high-speed wired internet is unavailable. The office must support cloud-based applications, video conferencing, and secure communication with the headquarters. The company has a limited budget for connectivity but requires uninterrupted business operations. As a network engineer, recommend a suitable internet connection, networking devices, and security mechanisms. Justify your recommendations by considering bandwidth limitations, reliability, cost, and future scalability

- A university plans to provide campus-wide wireless internet access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support a large number of simultaneous users while minimizing signal interference and ensuring secure access. Due to budget constraints, only a limited number of wireless access points can be deployed. Analyze the requirements and propose an efficient wireless network design, including access point placement, frequency band selection, and authentication methods. Explain how your solution balances coverage, performance, security, and cost.

- A financial institution is upgrading its internal computer network to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The organization must comply with strict security policies without significantly affecting network performance or increasing operational costs. Design a secure network architecture by selecting appropriate network segmentation techniques, firewall deployment, authentication methods, and intrusion detection mechanisms. Justify your design by considering the constraints of security, performance, compliance, and budget.

Question 1 – Remote Rural Branch Office

1. Problem and Constraints

The company is opening a branch in a remote rural location where high-speed wired Internet is unavailable. The office must support cloud applications, video conferencing, and secure communication with headquarters. The solution must have limited cost, reliable connectivity, sufficient bandwidth, uninterrupted operation, and future scalability.

2. Recommended Internet Connection

Use 4G/5G wireless broadband as the primary Internet connection because wired high-speed Internet is unavailable. A satellite connection can be considered where cellular coverage is poor.

- 4G/5G provides suitable bandwidth.

- Supports cloud-based applications.

- Supports video conferencing.

- Faster deployment than wired connectivity.

- Suitable for remote locations.

- Satellite provides an alternative where mobile coverage is unavailable.

3. Network Devices

Use the following devices:

- 4G/5G router – provides Internet connectivity.

- Managed switch – connects office devices.

- Wireless access point – provides Wi-Fi access.

- Firewall/router – controls network traffic.

- VPN gateway – provides secure headquarters communication.

4. Security Mechanisms

Use VPN encryption between the branch and headquarters to protect business communication.

- Firewall filters unwanted traffic.

- WPA2/WPA3 protects wireless access.

- Strong passwords prevent unauthorized access.

- User authentication controls network access.

- Regular updates improve device security.

5. Constraint-Based Solution

Bandwidth: 4G/5G provides suitable bandwidth for cloud services and video meetings.

Reliability: Use a secondary connection such as satellite or another cellular provider for backup.

Cost: Use 4G/5G as the primary connection because it avoids expensive wired infrastructure.

Security: Firewall, VPN, and secure Wi-Fi protect company communication.

Scalability: A managed switch and additional access points allow more users and devices to be added later.

6. Result

The proposed 4G/5G-based network with firewall, managed switch, secure Wi-Fi, and VPN provides a practical solution for the rural branch. It balances bandwidth, reliability, security, cost, and future scalability, supporting uninterrupted business operations.

Question 2 – University Campus Wireless Network

1. Problem and Constraints

The university needs campus-wide wireless access for more than 3,000 students across classrooms, laboratories, libraries, and hostels. The network must support many simultaneous users, reduce signal interference, provide secure access, and work within the limitation of having only a limited number of wireless access points.

2. Access Point Placement

Place access points at locations with high user density.

- Classrooms: APs near student areas.

- Laboratories: APs near working areas.

- Libraries: APs distributed for uniform coverage.

- Hostels: APs placed floor-wise.

- Avoid unnecessary overlapping coverage.

- Conduct a site survey before installation.

This allows the limited APs to provide maximum useful coverage.

3. Frequency Band Selection

Use both 2.4 GHz and 5 GHz bands.

- 2.4 GHz: Better coverage and wall penetration.

- 5 GHz: Higher speed and less interference.

- Prefer 5 GHz for high-density areas.

- Use non-overlapping channels where possible.

- Reduce overlapping channels between nearby APs.

This improves performance while reducing interference.

4. Authentication and Security

Use WPA2-Enterprise or WPA3-Enterprise with centralized authentication.

- Use individual student credentials.

- Avoid shared Wi-Fi passwords.

- Use 802.1X authentication.

- Separate student and administrative access.

- Monitor unauthorized access.

This provides secure access while supporting a large number of users.

5. Constraint-Based Solution

Coverage: Place APs according to user density.

Performance: Prefer 5 GHz and proper channel planning.

Security: Use enterprise authentication and individual credentials.

Cost: Optimize AP placement instead of installing unnecessary APs.

Capacity: Distribute users across APs to avoid overloading individual devices.

6. Result

The proposed wireless design balances coverage, performance, security, and cost by strategically placing limited APs, using suitable frequency bands, and applying enterprise authentication. This provides efficient wireless access for more than 3,000 students while minimizing interference.

Question 3 – Secure Network Architecture for Financial Institution

1. Problem and Constraints

The financial institution needs to improve cybersecurity while allowing employees to access sensitive customer information and online banking services. The design must satisfy strict security and compliance requirements without significantly reducing network performance or increasing operational costs.

2. Network Segmentation

Use VLAN-based network segmentation to separate different users and services.

- Employee systems use separate VLANs.

- Customer-data systems use a protected VLAN.

- Servers use a separate server network.

- Guest access remains isolated.

- Inter-VLAN communication is controlled.

This limits unauthorized access between different network sections.

3. Firewall Deployment

Deploy firewalls between the internal network, server network, and Internet.

- Filter incoming traffic.

- Filter outgoing traffic.

- Block unauthorized connections.

- Allow only required services.

- Monitor network traffic.

Firewall rules should follow the organization's security policies.

4. Authentication

Use strong authentication for employees accessing sensitive resources.

- Use username and password authentication.

- Apply multi-factor authentication (MFA).

- Use role-based access permissions.

- Give users only required privileges.

- Protect sensitive customer information.

5. Intrusion Detection

Use an Intrusion Detection System (IDS) to monitor suspicious network activity.

- Detect abnormal traffic.

- Identify possible attacks.

- Generate security alerts.

- Monitor important network segments.

- Support faster incident response.

6. Constraint-Based Solution

Security: VLANs, firewalls, MFA, and IDS provide layered protection.

Performance: Segmentation reduces unnecessary traffic.

Compliance: Access control and monitoring support security policies.

Budget: Use centralized security management and existing network infrastructure where possible.

7. Result

The proposed architecture combines network segmentation, firewall protection, strong authentication, and intrusion detection. It protects sensitive financial information while maintaining network performance and controlling operational costs. This provides a secure and manageable architecture suitable for the institution's requirements.

RUBRICS FOR CALCULATION

| Criteria | Weightage (%) | Exemplary (4) | Proficient (3) | Developing (2) | Beginning (1) |
| --- | --- | --- | --- | --- | --- |
| Understanding of Problem & Constraints | 20% | Clearly identifies all constraints and fully understands the problem requirements | Identifies most constraints with minor gaps in understanding | Identifies some constraints but misses key aspects | Shows little or no understanding of constraints or problem |
| Problem Decomposition | 15% | Breaks problem into logical, well-structured sub-problems effectively | Breaks problem into sub-parts with some logical flow | Attempts decomposition but lacks clarity or structure | No clear decomposition of the problem |
| Application of Constraints in Solution | 20% | Applies all constraints correctly and consistently in solution design | Applies most constraints correctly with minor errors | Applies some constraints but with noticeable errors | Fails to apply constraints appropriately |
| Logical Reasoning & Strategy | 15% | Uses strong, clear, and efficient reasoning strategies | Uses generally sound reasoning with minor gaps | Reasoning is partially correct but inconsistent | Reasoning is unclear or incorrect |
| Solution Accuracy & Feasibility | 15% | Solution is fully correct, feasible, and optimized | Solution is mostly correct and feasible | Solution has errors or is partially feasible | Solution is incorrect or not feasible |
| Creativity & Innovation | 5% | Demonstrates highly innovative and unique approach | Shows some creativity in approach | Limited creativity shown | No creativity; relies on basic or copied ideas |
| Clarity of Presentation | 10% | Solution is clearly presented with excellent organization and explanation | Presentation is clear with minor issues | Presentation lacks clarity or organization | Poorly presented and difficult to understand |