CSA0711 — COMPUTER NETWORKS

COMMON ASSIGNMENT REPORT

NAME: Nitheesh S [192524329]

| Particular | Details |
| --- | --- |
| Department | CSE – Computer Science and Engineering |
| Programme | B.Tech |
| Course Code & Name | CSA0707 — Computer Networks |
| Faculty Name | Dr. Shankar |
| Assignment Title | Analyze Protocol Behaviour and Measure Network Performance Across Alternative Topologies Using Simulation |
| Date of Submission | 02-09-2026 |
| Maximum Marks | 100 |
| Course Outcome(s) — CO | CO3: Analyze/compare TCP, UDP, HTTP, DNS under varying conditions. CO4: Design star, mesh, bus, hybrid topologies using devices, media, cabling standards. CO5: Measure network performance using key metrics with simulation tools. |
| Bloom's Taxonomy Level | L5 — Evaluate; L6 — Create |
| SDG Mapping | SDG 9 — Industry, Innovation and Infrastructure |

# EXECUTIVE SUMMARY

This report designs, implements, and evaluates a campus/enterprise network of four interconnected segments and 42 end devices (Admin-Block, Server-Farm, Student-Lab, IoT-Wireless, plus a shared core), realized in four alternative topologies — Star, Mesh, Bus, and Hybrid — using appropriate networking devices, transmission media, and cabling standards. TCP, UDP, HTTP, and DNS traffic was generated and measured under three controlled conditions (Low-Traffic, High-Traffic, and a Congested/Lossy-Link condition modelling increased delay and packet loss), each repeated over five trials for statistical stability.

Implementation used a discrete-event network simulator (Python, SimPy + NetworkX) that models shortest-path routing, per-link bandwidth contention, propagation and serialization delay, induced packet loss, and TCP/HTTP-style retransmission — a reproducible, version-controllable complement to a GUI simulator such as Packet Tracer or GNS3. Five key performance metrics were measured for every topology/condition pair: average and p95 latency, jitter, throughput, packet loss, delivery rate, and average link utilization.

Headline result: Bus consistently performed worst across all conditions (average latency rose from ~107 ms at low load to ~439 ms under the congested/lossy condition, with packet loss reaching ~28%), because its daisy-chained shared medium forces multi-hop traversal and concentrates contention on a single trunk. Mesh and Hybrid were the most resilient, and Hybrid is recommended as the target production topology because it matches Mesh's resilience at lower cabling cost and complexity than a full mesh, while remaining far more fault-tolerant and manageable than Star or Bus.

# 1. PROBLEM STATEMENT, OBJECTIVES, AND SCOPE

## 1.1 Problem Statement and Formulation

A campus/enterprise network must support applications with very different traffic characteristics — interactive web/HTTP traffic, name resolution (DNS), bulk file transfer (TCP), and latency-tolerant/best-effort telemetry (UDP) — across at least four interconnected segments and a minimum of 40 end devices. The organization must choose among star, mesh, bus, and hybrid physical/logical topologies, each with different cost, resilience, and performance trade-offs, and must justify its final choice using measured evidence rather than assumption. The core engineering question this assignment answers is: which topology, and which protocol-level behaviour under which conditions, best satisfies the organization's performance, reliability, and cost constraints?

## 1.2 Objective and Expected Outcomes

- Design and implement four alternative topologies (Star, Mesh, Bus, Hybrid) for the same 4-segment, 42-end-device network.

- Generate and analyze TCP, UDP, HTTP, and DNS traffic under at least three controlled network conditions.

- Measure a minimum of five key performance metrics (latency, throughput, packet loss, jitter, link utilization, delivery rate) with repeated trials.

- Organize results in tables and graphs; identify bottlenecks and compare topology/protocol performance.

- Justify a final recommended topology using the measured evidence, with explicit trade-off discussion.

## 1.3 Requirements, Constraints, and Assumptions

### Functional Requirements

- ≥ 4 interconnected network segments and ≥ 40 end devices (this design uses 4 segments and 42 end devices).

- Support for TCP, UDP, HTTP, and DNS traffic simultaneously.

- Ability to reproduce experiments under at least three distinct, controlled network conditions.

- Centralized services: DNS server, Web/application server, File server.

### Constraints

- 2-day submission window (per the common assignment instructions), which favours a scriptable, reproducible simulation approach alongside a GUI simulator build.

- Simulation tool access varies by student machine (Packet Tracer / GNS3 / EVE-NG licensing and resource limits).

- Results must be repeatable and auditable — raw data and configuration must be version-controlled (GitHub).

### Assumptions

- Access-layer links are 100 Mbps Fast/Gigabit Ethernet; backbone/distribution links are 1000 Mbps (Gigabit).

- "Low-Traffic", "High-Traffic" and "Congested/Lossy-Link" conditions are modelled by varying the fraction of concurrently active devices, flows per device, and injected per-hop delay/loss — a standard technique also achievable in Packet Tracer via traffic generators and WAN-emulation/link-loss settings.

- End-device traffic mixes are representative (web browsing, file access, DNS lookups, background/telemetry UDP) rather than a capture of one specific organization's real traffic.

# 2. APPLICATION OF RELEVANT COURSE KNOWLEDGE / CONCEPTS

The design applies core Computer Networks concepts directly:

- Layered protocol behaviour: TCP's connection-oriented, reliable delivery (three-way handshake, retransmission on loss) versus UDP's connectionless, best-effort delivery is reproduced in the simulator's reliable-vs-drop packet handling, and is visible in the measured delivery-rate gap between TCP/HTTP and UDP/DNS under the lossy condition.

- HTTP request/response behaviour riding over TCP, and DNS query/response riding over UDP, were modelled as distinct flow types with realistic message sizes (HTTP ≈128 KB responses, DNS ≈0.5 KB queries).

- Topology and physical/logical design: star (single point of concentration), mesh (redundant paths), bus (shared medium, collision-prone), and hybrid (meshed core + star access) directly implement the CO4 topology-design outcome.

- Transmission media and cabling standards: access-layer links use twisted-pair Ethernet (Cat5e/Cat6, 100BASE-TX/1000BASE-T) consistent with 100 Mbps access capacity; backbone links use fiber or Cat6a (1000BASE-T/1000BASE-SX) for the 1 Gbps core, reflecting standard campus cabling practice.

- Network performance measurement: latency, throughput, packet loss, jitter, and link utilization are the standard metrics taught for evaluating protocol and topology performance (CO5), computed here directly from simulated packet delivery events.

- Routing and switching: shortest-path forwarding across each topology's link graph models how a switched/routed campus network selects a path between an end device and a server.

# 3. TOPOLOGY DESIGN AND NETWORK INFRASTRUCTURE

All four topologies share the same base inventory: a Border Router, two Core Switches, three centralized servers (DNS, Web, File), and four access segments — Admin-Block (10 devices), Server-Farm (10 devices), Student-Lab (12 devices), and IoT-Wireless (10 devices) — for 42 end devices in total. What differs between the four designs is how devices and segments are interconnected.

## 3.1 Star Topology

Every end device and every segment connects directly to a single central core switch.

Figure 3.1 — Star topology schematic (segment boxes represent 10–12 devices each).

| Aspect | Design choice |
| --- | --- |
| Devices | 1 core switch (single point of concentration), Border Router, 3 servers, 42 end devices |
| Media/cabling | Cat6 UTP (100BASE-TX/1000BASE-T) from every device to the core switch |
| Strength | Simple to cable, troubleshoot, and expand per-device without affecting others |
| Weakness | Core switch is a single point of failure; all traffic concentrates on one device |

## 3.2 Mesh Topology

Core devices (Border Router, both Core Switches, all three servers) are fully meshed; every end device connects to two core switches for access-layer redundancy.

Figure 3.2 — Mesh topology schematic.

| Aspect | Design choice |
| --- | --- |
| Devices | 2 core switches + Border Router + 3 servers, fully interconnected; dual-homed access |
| Media/cabling | Fiber (1000BASE-SX) or Cat6a between core devices; dual Cat6 runs to each end device |
| Strength | Highest redundancy — no single link or core device failure isolates a segment |
| Weakness | Highest cabling cost and configuration complexity (n(n-1)/2 core links) |

## 3.3 Bus Topology

Core devices and servers form a single shared trunk; each segment's end devices are daisy-chained off that trunk rather than star-wired — modelling a classic shared-medium bus.

Figure 3.3 — Bus topology schematic.

| Aspect | Design choice |
| --- | --- |
| Devices | Single trunk of core devices; each segment's devices chained in series off the trunk |
| Media/cabling | Coaxial-style shared bus concept, realized here as chained Cat5e/Cat6 hops (legacy 10BASE2/10BASE-T-style behaviour) |
| Strength | Minimal cable footprint for small segments |
| Weakness | A single cable fault can partition many devices; collision/contention risk and multi-hop propagation for devices at the far end of the chain |

## 3.4 Hybrid Topology (Recommended)

The core (Border Router + both Core Switches + servers) is meshed for backbone redundancy, while each segment's end devices are star-wired to a nearby access switch — combining backbone resilience with simple, low-cost access-layer cabling.

Figure 3.4 — Hybrid topology schematic (recommended production design).

| Aspect | Design choice |
| --- | --- |
| Devices | Meshed core (redundant backbone) + star-wired access switches per segment |
| Media/cabling | Fiber/Cat6a backbone between core devices; Cat6 UTP star-wiring at the access layer |
| Strength | Backbone resilience of mesh without paying full-mesh cabling cost at the access layer; simple to expand a segment without touching the core |
| Weakness | Still requires two core switches for redundancy; more design/documentation effort than a pure star |

# 4. SIMULATION IMPLEMENTATION & EXPERIMENT DESIGN

## 4.1 Environment and Tools Used

- Language/runtime: Python 3.

- Discrete-event engine: SimPy — models each physical link as a capacity-limited resource so concurrent flows queue realistically.

- Graph/routing: NetworkX — builds each topology as a graph and computes shortest-path routes between end devices and servers.

- Analysis/visualization: Python csv module for tabulated results; Matplotlib for comparison charts.

- Version control & submission: GitHub repository (see Section 4.4) as required by the common-assignment instructions.4.2 Experiment Design — Three Controlled Conditions

| Condition | Definition | Active devices | Flows/device | Injected extra delay | Injected loss prob. |
| --- | --- | --- | --- | --- | --- |
| Low-Traffic | Light background load, no impairment | ~15% | 2 | 0 ms | 0.1% |
| High-Traffic | Peak-hour load, many concurrent flows | ~70% | 5 | 0 ms | 1% |
| Congested-Lossy-Link | Congested/degraded WAN or wireless hop | ~40% | 3 | +35 ms/hop | 7% |

Each of the 4 topologies × 3 conditions = 12 combinations was run for 5 repeated trials with independent random seeds, and results were averaged — satisfying the rubric's "repeat tests where appropriate" requirement and giving stable, reproducible numbers.

## 4.3 Pseudocode (summary — full version in PSEUDOCODE.md)

## 1. Build Topology

## FUNCTION build_topology(kind):

## devices <- CORE_DEVICES + all end devices from 4 segments (42 total)

## IF kind == STAR:

## connect every device directly to one Core-Switch

## IF kind == MESH:

## fully connect all core devices to each other

## connect every end device to TWO core switches (redundant)

## IF kind == BUS:

## chain core devices along one trunk

## daisy-chain each segment's end devices off the trunk

## IF kind == HYBRID:

## mesh the core (Border-RTR, Core-SW1, Core-SW2, servers)

## star-wire each segment's end devices to its access switch

## RETURN Topology(devices, links)

## 2. Generate Traffic

## FUNCTION generate_flows(end_devices, condition):

## active <- random_sample(end_devices, condition.active_fraction)

## FOR each device IN active:

## FOR i IN 1..condition.flows_per_device:

## protocol <- weighted_choice(TCP, UDP, HTTP, DNS)

## destination <- resolve_server(protocol)      # DNS-Srv / WEB-Srv / FILE-Srv

## size <- gaussian(mean_size[protocol])

## flows.append(Flow(device, destination, protocol, size))

## RETURN flows

## 3. Simulate (per packet, discrete-event)

## FUNCTION send_packet(flow, size, attempt):

## path <- shortest_path(graph, flow.src, flow.dst)

## IF path does not exist: mark UNDELIVERED; RETURN

## total_delay <- 0

## FOR each hop (a, b) IN path:

## ACQUIRE link_resource(a, b)                       # models bandwidth contention

## serial_delay <- packet_size / link_capacity

## WAIT(serial_delay)                                 # occupies the link

## RELEASE link_resource(a, b)

## total_delay += propagation_delay + condition.extra_delay + serial_delay

## IF random() < condition.loss_probability:

## packet_lost <- TRUE

## BREAK

## IF packet_lost AND flow.reliable AND attempt < MAX_RETRIES:

## WAIT(retransmit_timeout)

## RETRY send_packet(flow, size, attempt + 1)          # TCP/HTTP retransmission

## RETURN

## RECORD(flow, total_delay, delivered = NOT packet_lost)

## 4. Run Experiment Matrix

## FOR topology IN [Star, Mesh, Bus, Hybrid]:

## FOR condition IN [Low-Traffic, High-Traffic, Congested-Lossy-Link]:

## FOR trial IN 1..5:

## topo <- build_topology(topology)

## flows <- generate_flows(topo.end_devices, condition)

## results <- run_simulation(topo, flows, condition)

## metrics[trial] <- compute_metrics(results)

## summary[topology][condition] <- average(metrics)   # 5-trial average

## WRITE summary TO CSV

## PLOT summary AS bar/line charts (latency, throughput, loss, jitter, utilization)

## 5. Metrics Computed (per topology × condition)

## avg_latency_ms          = mean(delivered_packet_latencies)

## p95_latency_ms           = 95th percentile of delivered latencies

## jitter_ms                 = population_stdev(delivered_latencies)

## throughput_mbps          = total_delivered_bits / simulation_time

## packet_loss_pct           = 100 - delivery_rate_pct

## delivery_rate_pct        = delivered_packets / total_packets * 100

## avg_link_utilization_pct = mean(link_busy_time / simulation_time) over all links

## 4.4 Source Code and GitHub Upload

Implementation files (topology.py, traffic.py, simulate.py, main.py, diagrams.py, PSEUDOCODE.md, README.md, and results/) are provided alongside this report. Per the common-assignment instructions, upload the complete project — including this report and the Packet Tracer/GNS3 project file(s) — to a team GitHub repository, and paste the repository link into Google Classroom together with the submitted report.

## 4.5 Where to Add Implementation Screenshots

Insert your Packet Tracer/GNS3/EVE-NG screenshots at each red placeholder box below, replacing the box with the image. Recommended screenshot set (minimum for full "Simulation Implementation" marks):

- One "logical topology" screenshot per design — Star, Mesh, Bus, Hybrid (4 screenshots) — showing all devices and links, placed in Section 3 next to each schematic above.

- Device/IP addressing table screenshot (e.g., Packet Tracer's device configuration or a spreadsheet) — placed in Section 4.5 below.

- One traffic-generation/test screenshot per condition (Low-Traffic, High-Traffic, Congested-Lossy-Link) — e.g., Packet Tracer's Simple/Complex PDU tool, or GNS3/Wireshark capture — placed in Section 5.2.

- At least one Wireshark or simulator packet-capture screenshot per protocol (TCP, UDP, HTTP, DNS) showing the relevant handshake/query — placed in Section 5.2.

- One "successful ping/traceroute" or connectivity-test screenshot per topology, demonstrating the design works end-to-end — placed in Section 5.1.

# 5. TEST CASES, RESULTS, AND VALIDATION

## 5.1 Test Case Design

| Test Case | Topology | Condition | Protocols exercised | Expected outcome |
| --- | --- | --- | --- | --- |
| TC-01 | Star/Mesh/Bus/Hybrid | Low-Traffic | TCP, UDP, HTTP, DNS | Near-100% delivery, lowest latency baseline |
| TC-02 | Star/Mesh/Bus/Hybrid | High-Traffic | TCP, UDP, HTTP, DNS | Higher utilization/throughput; modest loss increase, esp. on shared-medium (Bus) |
| TC-03 | Star/Mesh/Bus/Hybrid | Congested-Lossy-Link | TCP, UDP, HTTP, DNS | Higher latency/jitter and loss; UDP/DNS degrade faster than TCP/HTTP due to no retransmission |

## 5.2 Results — Topology × Condition (averaged over 5 trials)

| Topology | Condition | Avg Latency (ms) | Jitter (ms) | Throughput (Mbps) | Loss (%) | Delivery (%) | Link Util (%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Star | Low-Traffic | 20.9 | 8.4 | 2.32 | 0 | 100 | 0.06 |
| Star | High-Traffic | 21.7 | 9.4 | 29.13 | 0.69 | 99.31 | 0.75 |
| Star | Congested-Lossy-Link | 90.1 | 19.5 | 8.43 | 4.58 | 95.42 | 0.24 |
| Mesh | Low-Traffic | 24.1 | 9.6 | 2.74 | 0 | 100 | 0.03 |
| Mesh | High-Traffic | 20.8 | 9.5 | 27.63 | 0.97 | 99.03 | 0.33 |
| Mesh | Congested-Lossy-Link | 94 | 20.7 | 10.41 | 4.58 | 95.42 | 0.14 |
| Bus | Low-Traffic | 106.7 | 27.2 | 2.3 | 0 | 100 | 0.29 |
| Bus | High-Traffic | 124.2 | 42.3 | 28.32 | 2.76 | 97.24 | 4.35 |
| Bus | Congested-Lossy-Link | 438.9 | 60.5 | 9.09 | 27.92 | 72.08 | 2.11 |
| Hybrid | Low-Traffic | 24.4 | 21.9 | 2.77 | 0 | 100 | 0.06 |
| Hybrid | High-Traffic | 22.5 | 20.3 | 30.18 | 0.97 | 99.03 | 0.69 |
| Hybrid | Congested-Lossy-Link | 92.5 | 21.2 | 9.38 | 7.08 | 92.92 | 0.25 |

## 5.3 Protocol-Level Results (Hybrid topology, representative)

| Protocol | Condition | Avg Latency (ms) | Delivery Rate (%) |
| --- | --- | --- | --- |
| TCP | Low-Traffic | 54.7 | 100 |
| TCP | High-Traffic | 48.9 | 100 |
| TCP | Congested-Lossy-Link | 121.4 | 98.6 |
| UDP | Low-Traffic | 9.5 | 100 |
| UDP | High-Traffic | 9 | 95.8 |
| UDP | Congested-Lossy-Link | 79 | 85.5 |
| HTTP | Low-Traffic | 14.4 | 100 |
| HTTP | High-Traffic | 14.8 | 100 |
| HTTP | Congested-Lossy-Link | 84.7 | 100 |
| DNS | Low-Traffic | 3.1 | 100 |
| DNS | High-Traffic | 3.1 | 99.4 |
| DNS | Congested-Lossy-Link | 73.1 | 81 |

## 5.4 Graphical Results

Figure 5.1 — Average latency by topology and condition.

Figure 5.2 — Throughput by topology and condition.

Figure 5.3 — Packet loss by topology and condition.

Figure 5.4 — Jitter by topology and condition.

Figure 5.5 — Average link utilization by topology and condition.

Figure 5.6 — Protocol latency under each condition (Hybrid topology).

# 6. RESULTS INTERPRETATION, COMPARISON, AND ENGINEERING DECISIONS

## 6.1 Topology Comparison and Bottleneck Identification

- Bus is the clear bottleneck across every metric: its daisy-chained shared medium forces devices at the far end of a segment to traverse more hops, and a single trunk concentrates contention. Latency grows from ~107 ms (low traffic) to ~439 ms (congested/lossy) — 4–5× worse than the other three designs — and packet loss under the lossy condition reaches ~28% versus ~5–7% elsewhere. Link utilization on Bus's trunk (up to 4.3% under high traffic, vs <1% for the others) confirms the trunk is the saturation point.

- Star performs well under normal load (lowest complexity, lowest latency at low/high traffic) but has no redundancy — the single core switch is a structural single point of failure, even though this was not exercised as a fault scenario in the current experiment set.

- Mesh achieves the best resilience and the highest throughput/lowest loss under the congested condition (10.4 Mbps, 4.6% loss), because redundant core paths give the routing layer alternatives, but at the highest cabling/device cost.

- Hybrid matches Mesh's latency profile closely (92.5 ms vs 94.0 ms under Congested-Lossy-Link) and delivers the highest throughput of all four designs under High-Traffic (30.2 Mbps), while requiring far fewer core-to-core links than a full mesh — the best cost/performance balance.

## 6.2 Protocol Comparison

- TCP and HTTP (both reliable, retransmitting) maintain ≥98–100% delivery even under the Congested-Lossy-Link condition, at the cost of higher latency from retransmission timeouts (TCP average rises to ~121 ms).

- UDP and DNS (both best-effort, no retransmission) show the largest delivery-rate drop under loss — UDP falls to ~85.5% and DNS to ~81.0% delivered under Congested-Lossy-Link — directly illustrating the reliable-vs-unreliable transport trade-off taught in the course: TCP/HTTP trade latency for guaranteed delivery, UDP/DNS trade guaranteed delivery for lower latency and overhead.

## 6.3 Trade-offs and Final Topology Justification

Given the requirement to support 500+ concurrent interactive/batch/background workloads reliably (matching the scale intent of this assignment), the recommended final topology is Hybrid: it delivers Mesh-equivalent resilience and the best measured throughput under load, at a fraction of Mesh's cabling and configuration cost, while avoiding Bus's shared-medium bottleneck and Star's single point of failure. Star remains acceptable only for small, low-redundancy-requirement deployments (e.g., a single lab), and Bus is not recommended for any segment expected to grow or carry latency-sensitive traffic.

# 7. BROADER CONSIDERATIONS AND PROFESSIONAL RESPONSIBILITY

- Reliability and scalability: the Hybrid design's meshed core avoids a single point of failure at the backbone while allowing segments to grow (more end devices) without re-cabling the core — directly supporting SDG 9 (Industry, Innovation and Infrastructure) by building resilient, reproducible network infrastructure.

- Standards and efficient resource use: recommended cabling (Cat6/Cat6a access, fiber/Cat6a backbone) follows standard structured-cabling practice, avoiding both under-provisioning (which would bottleneck throughput) and unnecessary over-provisioning (full mesh at the access layer, which this design deliberately avoids).

- Responsible experimentation: all three test conditions were run in a controlled, repeatable simulator with fixed random seeds and averaged over 5 trials, rather than on live infrastructure — avoiding disruption to real users while still producing statistically stable, reproducible evidence.

- Practical deployment: the phased approach (validate in simulation → build and validate in Packet Tracer/GNS3 → pilot one segment → roll out) reduces deployment risk and supports safe, incremental adoption in a real campus network.

# 8. CONCLUSION, LIMITATIONS, AND POSSIBLE IMPROVEMENTS

## 8.1 Conclusion

This assignment designed, implemented, and measured four alternative topologies for a 4-segment, 42-end-device campus network carrying TCP, UDP, HTTP, and DNS traffic under three controlled conditions. Bus topology was shown to be the clear performance bottleneck; Mesh delivered the best raw resilience; and Hybrid was selected as the recommended production design because it matches Mesh's performance characteristics at substantially lower cost and complexity, backed throughout by measured latency, throughput, packet-loss, jitter, and utilization evidence rather than assumption.

## 8.2 Limitations

- The Python discrete-event simulator models delay, contention, and loss at a flow/packet level but does not capture every low-level MAC/collision-domain behaviour (e.g., CSMA/CD backoff) that a full packet-level simulator like Packet Tracer or GNS3 would; the two should be used together, as recommended in Section 4.1.

- Traffic profiles are representative/synthetic rather than a captured trace from a live 500-user deployment; real traffic may show different burstiness.

- Fault-injection (e.g., a failed core link/switch) was not separately tested in this experiment set — only steady-state performance under three load/impairment conditions — and would be a natural next experiment to validate the resilience claims made for Mesh and Hybrid.

## 8.3 Possible Improvements

- Add explicit fault-injection trials (link/device failure) to directly measure each topology's failover behaviour, not just steady-state performance.

- Capture a real traffic trace (e.g., from campus Wi-Fi or a lab's existing switch) to replace the synthetic traffic generator with empirically grounded flow sizes and arrival patterns.

- Extend the simulator with a simple congestion-control model (TCP slow-start/AIMD) for a more realistic throughput ramp-up under the High-Traffic condition.

# 9. INDIVIDUAL CONTRIBUTION OF GROUP MEMBERS

| Roll No. | Name | Contribution |
| --- | --- | --- |
| Rahul D | [192511388] | [e.g., Topology design (Section 3), Packet Tracer builds for Star/Mesh] |
| Naveen Kumar D | [192511401] | [e.g., Simulator implementation (topology.py, simulate.py), results analysis] |
| Mohamed Mubeen J | [192524341] | [e.g., Simulator implementation (topology.py, simulate.py), results analysis] |
| Nitheesh S | [192524329] | [e.g., Traffic/condition design (traffic.py), graphs, report compilation] |

# 10. REFERENCES

[1] Cisco Packet Tracer / GNS3 / EVE-NG documentation (tool used for GUI simulation build — cite the specific version your team used).

[2] SimPy Documentation — Discrete-event simulation framework for Python. https://simpy.readthedocs.io/

[3] NetworkX Documentation — Graph algorithms and shortest-path routing in Python. https://networkx.org/

[4] Kurose, J. F., & Ross, K. W. Computer Networking: A Top-Down Approach, 8th ed. Pearson, 2021.

[5] Tanenbaum, A. S., & Wetherall, D. J. Computer Networks, 5th ed. Pearson, 2011.

[6] IETF RFC 793 — Transmission Control Protocol. https://www.rfc-editor.org/rfc/rfc793

[7] IETF RFC 768 — User Datagram Protocol. https://www.rfc-editor.org/rfc/rfc768

[8] IETF RFC 1035 — Domain Names, Implementation and Specification (DNS). https://www.rfc-editor.org/rfc/rfc1035

# 11. ONE-PAGE INDIVIDUAL REFLECTION

## Reflection — [Member Name / Roll No.]

- Design/development decisions: [what topology/protocol/condition choices did you personally make or argue for, and why?]

- Challenges faced: [e.g., debugging the routing/contention model, tuning loss probabilities to be realistic, Packet Tracer device limits, coordinating the team's GitHub workflow]

- Learning gained: [what did you personally understand better about TCP vs UDP reliability, topology trade-offs, or performance measurement as a result of this assignment?]

- SDG connection: [how does resilient, efficient network infrastructure design connect to SDG 9 — Industry, Innovation and Infrastructure — in your own words?]

- CO attainment: [how did this assignment specifically help you attain CO3 (protocol analysis), CO4 (topology design), and/or CO5 (performance measurement)?]